TURBulence METHODS

Microstructure profilers, ADCPs, Wire-Walker, Moored thermistors, Eddy correlation

Table of Contents

1.	Basic turbulence theory	4

1.1.	What is turbulence?	4

1.2.	Turbulence isotropy	4

1.3.	Conservation equations for turbulent flows	4

1.4.	Turbulence intensity parameters	4

1.4.1.	Definitions	4

1.4.2.	Approximate estimations (without microstructure measurements)	4

1.4.3.	In situ turbulence measurements	4

1.5.	Turbulent fluxes (eddy diffusivity)	5

1.5.1.	Estimation from turbulence intensity	5

1.5.2.	Other estimations	5

1.6.	Mixing efficiency	5

1.7.	Taylor’s Frozen Hypothesis	5

1.8.	Energy cascade	6

1.9.	Characteristics length scales	6

1.10.	Characteristics wavenumbers	6

2.	Microstructure profilers	7

2.1.	Introduction	7

2.2.	Instruments and sensors	7

2.3.	Parameters for the data analysis	8

2.4.	Data	8

2.4.1.	Raw data (Level 0)	8

2.4.2.	Imported data (Level 1)	8

2.4.3.	Processed data (Level 2)	8

2.5.	Scripts structure, overview of the analysis	8

2.6.	STEP 1 – Import the raw data (odas_p2mat_print)	10

2.7.	STEP 2 – Configuration file update (get_config)	10

2.8.	STEP 3 – Profile extraction (get_profile)	10

2.9.	STEP 4 – FP07 calibration (cal_FP07_in_situ_EPFL)	10

2.10.	STEP 5 – Odas analysis (quick_look)	10

2.11.	STEP 6 – Salinity & density (compute_rho_salinity)	10

2.12.	STEP 7 – Pressure correction (correct_pressure)	10

2.13.	STEP 8 – Thorpe displacements (resolve_turbulence)	11

2.14.	STEP 9 – Bins creation (resolve_turbulence)	12

2.14.1.	Pressure of the bins	12

2.14.2.	Bin-averaged quantities	12

2.14.3.	Thorpe length scale	13

2.15.	STEP 10 – Filtering & despiking (resolve_turbulence)	13

2.15.1.	Shear signal despiking	13

2.15.2.	Shear signal filtering	13

2.15.3.	Accelerometer signal despiking	14

2.15.4.	Selection of accelerometer signal for noise filtering	14

2.16.	STEP 11 – Diffusivity and viscosity (resolve_turbulence)	15

2.17.	STEP 12 – Vibration, inclination and acceleration flagging (resolve_turbulence)	15

2.17.1.	Vibration	15

2.17.2.	Inclination	16

2.17.3.	Acceleration	16

2.18.	STEP 13 – Shear spectral analysis (TKE_dis_spec)	16

2.18.1.	Shear signal detrending	18

2.18.2.	Power spectral density computation	18

2.18.3.	Denoised spectrum (Goodman)	19

2.18.4.	Spatial response correction	20

2.18.5.	Iterative calculation of ε	20

2.18.6.	Lost variance correction	20

2.18.7.	Spectral fit to recalculate ε	20

2.18.8.	Median Absolute Deviation (MAD) calculation	20

2.19.	STEP 14 – FP07 spectral analysis (gradT_dis_spec)	21

2.19.1.	Temperature gradient detrending	21

2.19.2.	Power spectral density computation	21

2.19.3.	Conversion from time to space	22

2.19.4.	Time response correction	22

2.19.5.	Noise spectrum	23

2.19.6.	Integration range	23

2.19.7.	Calculate χST from shear	24

2.19.8.	Fitting (to get χT and εT)	24

2.19.9.	Quality metrics	27

2.20.	STEP 15 – Spectral analysis flagging (resolve_turbulence)	27

2.21.	STEP 16 – Vertical eddy diffusivity (resolve_turbulence)	28

2.22.	STEP 17 – Ozmidov length scale (resolve_turbulence)	28

3.	ADCP	29

4.	Wire-walker	29

5.	Moored thermistors	29

6.	Eddy correlation	29

7.	Appendix A - Lists of variables for microstructure profilers	29

7.1.	Parameters	29

7.2.	Level 1 data	34

7.3.	Level 2 data	36

# Basic turbulence theory

See EPFL + ETHZ + Rockland courses on turbulence + e-book https://eaglepubs.erau.edu/introductiontoaerospaceflightvehicles/chapter/turbulent-flows/

## What is turbulence?

## Turbulence isotropy

## Conservation equations for turbulent flows

- Reynolds decomposition

- Turbulent Kinetic Energy equation & TKE budget  rate of dissipation + mixing

## Turbulence intensity parameters

### Definitions

- Dissipation rate of TKE (by viscous friction):

ε=7.5ν∂u´∂z2 [W/kg]

where ν is the water kinematic viscosity [m2/s]

It represents how fast kinetic energy is destroyed by viscous friction (conversion into heat) at Kolmogorov scale (~mm). ε increases with the gradient of velocity fluctuations.

- Dissipation rate of thermal variance (by molecular heat diffusion):

χ=6κT ∂T´∂z2 [°C2/s]

It represents how fast temperature fluctuations are erased my molecular heat diffusion at Batchelor scale (~10 um). χ increases with the gradient of temperature fluctuations.

### Approximate estimations (without microstructure measurements)

- ε from Thorpe scale  CTD

- ε from atmospheric forcing (only for surface layer)

- ε from log velocity profile (e.g., bottom boundary layer)

And χ from ε?

### In situ turbulence measurements

- ε,χ from shear profiles

  - The velocity gradient ∂u∂z is directly measured by the shear probe

  - More suitable for strong turbulence (oceans)

  - Give scales in spectrum  requires faster profiling speed

- ε,χ from microstructure temperature profiles:

  - The temporal gradient of temperature fluctuations ∂T´∂t is converted to a vertical gradient ∂T´∂z by using the Taylor-frozen hypothesis

  - Give scales in spectrum  required slower profiling speed to resolve small scales

  - More suitable for weak turbulence (lakes)

  - In practice, χ is estimated by integrating the temperature gradient spectrum with noise subtracted (Piccolroaz et al., 2021).

- ε from structure function  ADCP

- ε from χ  moored sensors (Senafi et al., 2025)

ε=N2χ2 Γ ∂T/∂z

## Turbulent fluxes (eddy diffusivity)

Vertical eddy diffusivity = turbulent diffusivity = diapycnal diffusivity (across density interfaces)

### Estimation from turbulence intensity

The vertical eddy diffusivity Kz can be estimated from turbulence intensity parameters:

- Osborn (1980) relationship: Kz=Γε/N2 (assuming a mixing efficiency Γ that could vary)

- Osborn & Cox (1972) model: Kz=0.5 χ∂T∂z-2

### Other estimations

In absence of turbulence measurements, Kz can be estimated from changes in stratification with the Flux-Gradient method (Powell & Jassby, 1974; Jassby & Powell, 1975) + add other methods (tracer, locally-solved heat equation)

## Mixing efficiency

The mixing efficiency Γ relating Kz to ε in the Osborn relationship vary between systems. A method to estimate Γ consists in measuring εS (from shear probes) and χT (from temperature probes) and combining the Osborn relationship with the Osborn-Cox model as (Piccolroaz et al., 2021):

Γ=0.5N2 χT∂T∂z-2εS-1

Other method from energy budget: see Wüest et al., 2000.

## Taylor’s Frozen Hypothesis

The Taylor’s Frozen Hypothesis assumes that turbulent structures move past the sensor without changing much in time, such that ∂X∂x≈-1U∂X∂t where U is the mean advection velocity of the turbulent structures (i.e., equals the vertical profiler velocity U=-W for x=z). To be valid, Taylor’s hypothesis requires that the turbulent structures are advected fast enough compared to their temporal evolution (i.e., U should be large enough) (Piccolroaz et al., 2021).

This hypothesis is required to convert temporal fluctuations measured by a sensor into spatial fluctuations required to compute wavenumber-dependent spectra. Even for shear probes that directly measure a spatial gradient, Taylor’s hypothesis is implicitly used to express the shear signal as a function of pressure/depth instead of time, i.e. to compute the spatial frequency used for the spectrum computation (function csd_odas).

## Energy cascade

Source: https://eaglepubs.erau.edu/introductiontoaerospaceflightvehicles/chapter/turbulent-flows/

Turbulent production (Reynold’s stress in the TKE equation) occurs at large scales, in the energy-containing range (Imboden & Wüest, 1995). This range contains the peak of the energy spectrum above.

Energy is transferred from large to smaller scales in the inertial subrange (i.e., energy cascade). Under steady state with isotropic and homogeneous turbulence without external input or loss of energy, the energy flux is constant in this range and the spectrum only depends on ε at the small-scale end. In this case, energy decreases with decreasing scales according to the -5/3 law (i.e., linear decrease on a log-log spectrum). This is only valid in non-stratified layers to get isotropic conditions (Imboden & Wüest, 1995).

Energy contained in velocity fluctuations is dissipated into heat by molecular viscosity at the smallest scales given by the Kolmogorov scale  which depends on ε: higher level of turbulence leads to dissipation at smaller scales. It is the dissipation range.

The energy spectrum of a passive scalar (i.e., temperature), can be further divided into other ranges. At length scales larger than the Kolmogorov scale but less than the energy-containing scale, the scalar spectrum follows the -5/3 similar to the velocity spectrum: it is the inertial-convective subrange. Between the Kolmogorov scale and the Batchelor scale, velocity fluctuations are reduced by viscosity but scalar fluctuations are not dissipated yet: it is the viscous-convective subrange (corresponding to the Batchelor spectrum) where the energy decreases inversely to the wavenumber (-1 law). At scales smaller than the Batchelor scale, scalar fluctuations are dissipated by molecular diffusion: it is the diffusive range.

## Characteristics length scales

See Imboden & Wüest (1995) and Wüest & Lorke (2003)

- Energy containing scale: characteristic length scale of the studied system (lake, mixed layer, etc.)

- Kolmogorov scale LK=ν3ε14: scale below which viscous forces overcome inertial forces which leads to velocity dissipation by molecular viscosity

- Batchelor scale LB=νD2ε14: similar as the Kolmogorov scale for a passive scalar (e.g., temperature), i.e. where fluctuations of the scalar dissipate by molecular diffusion. It is smaller than  LK.

- Taylor scale: see Notions doc

- Thorpe scale

- Ozmidov scale  LO=εN312: scale above which buoyancy forces overcome inertial forces, i.e. maximum vertical scale of eddies/overturns

Note that LO is often assumed to be proportional to the Thorpe scale LT (Dillon, 1982), yet such simple scaling may not be generally applicable (Smyth et al., 2001).

## Characteristics wavenumbers

- Kolmogorov wavenumber kK=ν3ε14 (not calculated ?): lowest limit of the wavenumber range for which viscous dissipation dominates (based on kinematic viscosity) = where spectral fitting can be done.

- Batchelor wavenumber kB=νDT2ε14>kK : lowest limit of the wavenumber range for which molecular thermal diffusion dominates over turbulence.

- kmin: minimum wavenumber for spectral integration, chosen as a small value (e.g., 0.1 cpm for shear, 1 cpm for temperature), also called k1 in the scripts

- kS=fkS*Pr-0.5kB: upper bound of the inertial-convective subrange, lower wavenumber limit according to Steinbuck et al. (2009), with fkS=0.04

- kAA=fAA/W: cut-off wavenumber of the anti-aliasing filter

- kn=fn/W: maximal wavenumber for calculations, with fn=0.9fAA

- kSNR: wavenumber above which noise dominates (i.e., SNR becomes lower than threshold SNRmin) : kSNR such that PSDkSNR≤SNRmin*SnkSNR with SNRmin=1.55 (Goto et al., 2018)

- kH: wavenumber above which the time response correction factor is below threshold value H<Hlim (with typically Hlim=0.1)

- kP=kB006q: wavenumber corresponding to the peak of the fitted theoretical spectrum, where kB00 is the Batchelor wavenumber that gives the maximum likelihood

- kL: lower wavenumber bound for spectral integration; computed in Piccolroaz et al. (2021) as kU=kminfor shear proves and  kU=maxkmin,kS for microstructure temperature

- kU: upper wavenumber bound for spectral integration; computed in Piccolroaz et al. (2021) as kU=minkn,0.76kK,150 cpm  for shear probes and  kU=minkSNR,kH,kB,kn for microstructure temperature

# Microstructure profilers

## Introduction

The method described in this section follows Piccolroaz, S., Fernández-Castro, B., Toffolon, M. et al. A multi-site, year-round turbulence microstructure atlas for the deep perialpine Lake Garda. Sci Data 8, 188 (2021). https://doi.org/10.1038/s41597-021-00965-0.

## Instruments and sensors

For principle of measurements and information on sensors, see Technical Note 005 - Converting shear-probe, thermistor and micro-conductivity signals into physical units.

List here differences between microCTD and VMP data:

- Vibrations: Ax, Ay, Az can either be measured from linear accelerometers (type=accel, units =m/s2) or from piezo-accelerometers (type=piezo, units = counts) according to the ODAS manual. The microCTD use piezo-accelerometers. The configuration file of the VMP use type=accel but the VMP manual indicates that piezo-accelerometers are installed.

- Inclination can be directed computed if present in channels apparently, otherwise it is computed as (see quick_look function with plot_kinematics option):

%-- low pass filter and estimate inclination

f_low_pass = 1; % in Hz

[b,a] = butter(2, f_low_pass /(fs_AA/2));

X_data = filtfilt(b,a,AA_linear(:,1:min(2,end))); % because we want rotation around y-axis

X_data = asind(X_data / 9.81); % inclination [degrees]

## Measurement considerations

List here the requirements for good measurements with shear probes and FP07 (or microcond)

- Profiler velocity:

  - Should be constant

  - Shear probes: Rueck et al. (2024) mentioned that velocity >0.3 m/s is usually enough be larger than current velocities and even mention good measurements down to 0.1 m/s. Piccolroaz et al. (2021) used ~0.7 m/s

- Turbulence intensity (ranges provided by AI, to be checked)

  - Shear: typically ε~10-10-10-4 W/kg

  - FP07: typically ε~10-12-10-7 W/kg (lower limit: see Scheifele et al., 2008)

  - Good match between FP07 and shear: ~10-10-10-7 (lower limit is close to 10-11 according to Scheifele et al., 2008)

## Instrument configuration

Each .P data file contains a configuration string that comes from the instrument configuration file (can be seen by opening the .P file with a text editor).

- If FP07 probes are used, the configuration needs to be updated after data acquisition to apply temperature calibration (parameters “beta_i” and “T_0” in the fast temperature channels). This will be done at STEP 4 (see Sect. 2.10).

- If shear probes are used, the probe sensitivities found in calibration sheets must be included in the calibration file (parameter “sens” in the shear channels), either before deployment (i.e., the measurements will be collected with the correct shear sensitivities) or after deployment (i.e., the shear measurements will be corrected at STEP 2 of the data processing, see Sect. 2.8).

- If a pressure offset must be applied for atmospheric pressure correction, the configuration must include the pressure offset (parameter “coef0” in the pressure channel).

## Parameters for the data analysis

The parameters are defined in the load_parameters_lakename function and listed in Appendix A, Sect. 7.1 (to be continued).

See TN039 (Sect. 4) + ODAS manual (odas_p2mat, quick_look) for more info on parameters & check if parameters are missing

## Data

### Raw data (Level 0)

The raw data consist of .P files that can contain several profiles. These files are imported to Matlab with the ODAS function odas_p2mat.

### Imported data (Level 1)

The ODAS function odas_p2mat extracts the data from the .P file. The Level 1 data is saved as a .mat (Level1/VMPxxx_.../L1_filename_direction.mat) and .nc file. The variables included in the .mat file is listed in Appendix A, Sect. 7.2.

### Processed data (Level 2)

The data produced by the script analyze_profiles.m is stored for each data file in Level2/VMPxxx_.../ L2_filename_direction_profnumber.mat and for each profile in a separate .nc file. It consists of the following variables:

- param (structure): parameters used for the analysis

- SLOW (cell array, one cell per profile): each cell contains the profile data as a structure, at the frequency of the CTD sensors (64 Hz, sampling interval of 0.0156 sec)

- FAST (cell array, one cell per profile): each cell contains the profile data as a structure, at the frequency of the microstructure sensors (512 Hz, sampling interval of 0.002 sec)

- BINNED (cell array, one cell per profile): each cell contains the profile data as a structure, organized in bins/segments (turbulence analysis from Piccolroaz et al., 2021)

- DISS_QL (cell array, one cell per profile): each cell contains the profile data as a structure, organized in bins/segments (ODAS turbulence analysis with quick_look)

The variables of the four cell arrays are listed in Appendix A, Sect. 7.2.

## Scripts structure, overview of the analysis

The structure of the main scripts is summarized in the two schematics below:

## STEP 1 – Import the raw data (odas_p2mat_print)

Refer to the variables that are obtained (Sect. 2.3.1)

## STEP 2 – Configuration file update (get_config)

The function performs the following steps:

- The configuration file from the *.P data file is extracted and saved as setup_namefile_original.cfg in the Level2 file-specific folder. If modify_cfg=False, this original configuration file will be used for the rest of the processing and the data file *_patched.P saved in the  Level2 file-specific folder is just a copy of the original data file.

- If modify_cfg=True, the configuration file is modified as follows:

  - If a configuration file has been specified in the parameters (param.cfg_file), all the configuration parameters from this file are imported. If the pressure correction method is set to “offset”, the pressure offset must be specified in the configuration file.

  - If param.cfg_file is not specified or empty, the shear sensitivities must be specified as parameters in load_parameters_lake_name.m as param.S_sh1 and param.S_sh2, otherwise the script will return an error. If the pressure correction method is set to “offset”, the offset must be specified as param.offset_P or entered manually while running get_config().

The modified configuration is patched to the data file and exported as setup_namefile_modified.cfg and *_patched.P in the  Level2 file-specific folder.

- If modify_cfg=True, the patched data file is imported with odas_p2mat().

## STEP 3 – Profile extraction (get_profile)

## STEP 4 – FP07 calibration (cal_FP07_in_situ_EPFL)

## STEP 5 – Odas analysis (quick_look)

Quick summary of the steps here, refer to ODAS Manual (quick_look, get_diss_odas…). Make a slide about quick_look?

The observed shear and acceleration cross-spectra are calculated in frequency space. The shear spectrum is then converted to wavenumber space using the Taylor’s frozen hypothesis.

The shear spectral analysis is done on the entire profile first, then on each bin (see while loop in get_diss_odas), defined by its duration diss_length as an argument of quick_look, converted to number of samples for get_diss_odas.

Scalar spectra are also computed with get_scalar_spectra_odas

## STEP 6 – Salinity & density (compute_rho_salinity)

## STEP 7 – Pressure correction (correct_pressure)

Need to check if the the pressure offset is correctly used:

- pmin is specified in parameters (minimum depth for profile, not pressure offset)  used in get_profile

- offset_P is specified in the parameters and included in the modified cfg file in get_config  does it mean that the pressure correction is done when profiles are imported again?

- after pressure correction, profiles are re-extracted with get_profile using corrected pressure (0 in the air) and pmin=0 (this part is unclear: why is it required to set pmin=0?)

## STEP 8 – Thorpe displacements (resolve_turbulence)

The Thorpe displacements are computed from the slow density profile (sgt), fast T1 and fast T2:

if ismember(info.prof_dir,'down')

[sort_rho, isd] = sort(sgt); % Sort slow density profile, from low to high density

displ = Ps - Ps(isd); % displacements [m] = distance between the depth i and the depth where the sorted density is measured

% Depth of sorted density value = current depth i - displacement i

[sort_uT1, ist1] = sort(T1f,'descend'); % Sort fast T1 profile, from high to low temperature

displuT1 = Pf - Pf(ist1);

[sort_uT2, ist2] = sort(T2f,'descend'); % Sort fast T2 profile, from high to low temperature

displuT2 = Pf - Pf(ist2);

[sort_T_JAC, ~] = sort(T_JAC,'descend'); % Sort JAC_T profile (not for Thorpe displacements), from high to low temperature

else

[sort_rho, isd] = sort(sgt,'descend'); % Sort slow density profile, from high to low density

displ = Ps - Ps(isd);

[sort_uT1, ist1] = sort(T1f,'ascend'); % Sort fast T1 profile, from low to high temperature

displuT1 = Pf - Pf(ist1);

[sort_uT2, ist2] = sort(T2f,'ascend');% Sort fast T2 profile, from low to high temperature

displuT2 = Pf - Pf(ist2);

[sort_T_JAC, ~] = sort(T_JAC,'ascend'); % Sort slow JAC_T profile (not for Thorpe displacements), from low to high temperature

end

% centered Thorpe scale

Ps_centered=Ps-displ*0.5; % Centred pressure values = pressure of the middle of the Thorpe segments (~center of overturns)

Lc=zeros(1,length(Ps)); count_c=zeros(1,length(Ps));

for i_Ps=1:length(Ps)

[~,i_c] = min(abs(Ps-Ps_centered(i_Ps))); % Closest pressure value to the centered pressure values

Lc(i_c) = Lc(i_c) + abs(displ(i_Ps));

count_c(i_c) = count_c(i_c) + 1;

end

Lc = Lc./count_c; Lc(count_c==0)=0; % Thorpe scale profile computed as averaged Thorpe displacement for depths with several overturns

SLOW.LTc = Lc';

SLOW.LTavg=sqrt(mean(displ.^2,"omitnan")); % Added usual Thorpe scale definition, one value per profile (T. Doda)

% Note: LTavg is similar to the averaged non zero LTc

A high value of Thorpe displacements at depth i means that the density at depth i had to be moved far way during the sorting process: it is an indication of overturns. Low displacements correspond to stratified conditions (i.e., the observed profile is very similar to the sorted profile).

## STEP 9 – Bins creation (resolve_turbulence)

### Pressure of the bins

%defines the pressure vector where to calculate

pres = info.pmin:info.dp:info.pmax; % Pressure of the bins center (space between bins: overlapping dp)

if ismember(info.prof_dir,'up') % For upward profiles, start the profile after the acceleration

idx75=find(Ps<0.75*info.pmax,1,'first'); % Index of the depth corresponding to 25% of the entire profile (75 % from the surface). Note: Ps is decreasing (upward profile).

[~,idx]=min(Ws(1:idx75)); % Get the minimum speed over these first 25% (not affected by wave-induced oscillations above)

n_pres = find(pres<=Ps(idx),1,'last'); % Maximal depth where the profile starts (for upward)

else

n_pres=length(pres); % Use pmax as the maximal depth for downward profile

end

BIN.pressure = pres; % Pressure of each bin [dbar]

% Use dpD instead of dp for bin thickness:

BIN.depth = pres_av(Ps,depth,pres,info.dpD,2.7); % Average depth for each bin [m]

### Bin-averaged quantities

BIN.temperature = pres_av(Ps,T_JAC,pres,info.dpD,2.7); % Average temperature for each bin [°C]

BIN.conductivity = 1000*pres_av(Ps,C_JAC,pres,info.dpD,2.7); % Average condcutivity for each bin [uS/cm]

BIN.salinity = 1000*pres_av(Ps,Ss,pres,info.dpD,2.7); % Average salinity for each bin [mg/l]

BIN.density = pres_av(Ps,sgt,pres,info.dpD,2.7); % Average density for each bin [kg/m3]

for i=1:length(pres) % Loop across the bins

% It is better to use FP07 because when the lake is homogeneous,

% the resolution of JAC-T is too low, resulting in grT=1E-16

BIN.avggradT1(i) = mean_grad(Pf,sort_uT1,pres(i),max(Thorpe(i),info.dpD));   % SEB: evaluated on the resorted T profile, over the largest length scale between bin size and Thorpe length

BIN.avggradT2(i) = mean_grad(Pf,sort_uT2,pres(i),max(Thorpe(i),info.dpD));   % SEB: evaluated on the resorted T profile, over the largest length scale between bin size and Thorpe length

if param.config.T1 && param.config.T2

BIN.grT12(i)=nanmean([BIN.avggradT1(i),  BIN.avggradT2(i)]); % Mean gradient between the two FP07

elseif ~param.config.T2 % Only T1

BIN.grT12(i)=BIN.avggradT1(i);

elseif ~param.config.T1 % Only T2

BIN.grT12(i)=BIN.avggradT2(i);

else % No FP07

BIN.grT12(i)=NaN(size(BIN.avggradT1(i)));

end

BIN.avggradT(i) = mean_grad(Ps,sort_T_JAC,pres(i),max(Thorpe(i),info.dpD));   % SEB: evaluated on the resorted T profile, over the largest length scale between bin size and Thorpe length

BIN.N2(i) = -9.81*mean_grad(Ps,sort_rho,pres(i),max(Thorpe(i),info.dpD))/1000;

end

% Use dpD instead of dp for bin thickness:

BIN.chlorophyll = pres_av(Pf,Chl_fast,pres,info.dpD,2.7);

BIN.turbidity = pres_av(Pf,Turb_fast,pres,info.dpD,2.7);

BIN.acc = -mean_grad_time(Pf,Wf,pres,info.dpD,timef);  % instrument acceleration [dbar.s^(-2)]

### Thorpe length scale

Average absolute Thorpe displacement in each bin:

Thorpe=sqrt(pres_av(Ps,displ.^2,pres,info.dpD,0)); % Thorpe length for each bin (average absolute displacement) from slow density profile [m]

BIN.LT = Thorpe;

BIN.LTuT1 = sqrt(pres_av(Pf,displuT1.^2,pres,info.dpD,0)); % Thorpe length scale from T1

BIN.LTuT2 = sqrt(pres_av(Pf,displuT2.^2,pres,info.dpD,0)); % Thorpe length scale from T2

## STEP 10 – Filtering & despiking (resolve_turbulence)

### Shear signal despiking

To remove spikes in the shear signal (e.g., due to collisions with particles), the ODAS function despike is used:

[sh1, ~, ~, ~ ] =  despike(sh1, info.despike_sh(1), info.despike_sh(2), fsf, round(info.despike_sh(3)*fsf));

if param.config.S2

[sh2, ~, ~, ~ ] =  despike(sh2, info.despike_sh(1), info.despike_sh(2), fsf, round(info.despike_sh(3)*fsf));

end

### Shear signal filtering

A 1st order Butterworth highpass filter is applied on the shear signal to remove low-frequency noise:

mW = (max(Pf)-min(Pf))/(max(timef)-min(timef)); % Mean speed

if info.k_HP_cut>0

f_HP_cut = info.k_HP_cut*mW; % cutoff frequency [s-1]

[bh,ah] = butter(1, f_HP_cut/(fsf/2), 'high'); % Defines a 1st order Butterworth highpass filter with cutoff frequency normalized by half of the sampling frequency (Matlab function)

if param.config.S1

sh1_hp = filter(bh, ah, sh1); % Filters the data to enhance frequencies above f_HP_cut and remove low-frequency noise (Matlab function)

sh1_hp = flipud(sh1_hp);

sh1_hp = filter(bh, ah, sh1_hp);

sh1_hp = flipud(sh1_hp);

else

sh1_hp=sh1;

end

if param.config.S2

sh2_hp = filter(bh, ah, sh2);

sh2_hp = flipud(sh2_hp);

sh2_hp = filter(bh, ah, sh2_hp);

sh2_hp = flipud(sh2_hp);

else

sh2_hp=sh2;

end

else

sh1_hp = sh1;

sh2_hp = sh2;

end

FAST.fast_S1_hp=sh1_hp;

FAST.fast_S2_hp=sh2_hp;

### Accelerometer signal despiking

Spikes are also removed from the piezo-accelerometer signals with the ODAS function despike:

% Despike the piezo-accelerometer signals

piezo_accel_num = size(AA,2);

if  ~isempty(AA) && info.despike_A(1) ~= inf

for probe = 1:piezo_accel_num % For each accelerometer direction

[AA(:,probe), ~, ~, ~]  = ...

despike(AA(:,probe),  info.despike_A(1), ...

info.despike_A(2), fsf, round(info.despike_A(3)*fsf));

end

end

### Selection of accelerometer signal for noise filtering

Keep one of the two accelerometer signals (Ax and Ay) for noise filtering as the one that is the most strongly correlated with the shear signal in the frequency domain (higher average spectral coherence):

%-- identify the piezo-accelerometer signal to be used for noise filtering

% Compute autospectra of the detrended signals of sh1, Ax and Ay (i.e., residuals with respect to linear fit)

PSDsh = csd_odas(detrend(sh1_hp),detrend(sh1_hp),1024,fsf,[],512,'linear');

PSDA1 = csd_odas(detrend(AA(:,1)),detrend(AA(:,1)),1024,fsf,[],512,'linear');

PSDA2 = csd_odas(detrend(AA(:,2)),detrend(AA(:,2)),1024,fsf,[],512,'linear');

% Compute cross-spectra sh1-Ax and sh1-Ay:

CSDshA1 = csd_odas(detrend(sh1_hp),detrend(AA(:,1)),1024,fsf,[],512,'linear');

[CSDshA2,fA] = csd_odas(detrend(sh1_hp),detrend(AA(:,2)),1024,fsf,[],512,'linear');

% Compute magnitude-squared coherence ranging from 0 to 1 and measuring how strongly

% accelerometer and shear signals are linearly related at each frequency

COHshA1 = abs(CSDshA1.^2./(PSDA1.*PSDsh));

COHshA2 = abs(CSDshA2.^2./(PSDA2.*PSDsh));

AAxy=AA; % store both signals

if nanmean(COHshA1)>nanmean(COHshA2)

AA = AA(:,1);

else

AA = AA(:,2);

end

## STEP 11 – Diffusivity and viscosity (resolve_turbulence)

For each bin i, compute bin-averaged thermal conductivity, thermal diffusivity, dynamic viscosity and kinematic viscosity:

jp = find(Pf>=pres(i)-info.dpD/2 & Pf<=pres(i)+info.dpD/2); % cell indices that are part of bin i in fast channels

jps = find(Ps>=pres(i)-info.dpD/2 & Ps<=pres(i)+info.dpD/2);   % cell indices that are part of bin i in slow channels

avT = nanmean(T_JAC(jps)); % Average temperature in bin i

avS = nanmean(Ss(jps)); % Average salinity

avsigmat = nanmean(sgt(jps)); % Average density

cond = SW_Conductivity(avT,'C',avS,'ppt'); % Thermal conductivity

BIN.ThermCond(i)=cond;

cp = SW_SpcHeat(avT,'C',avS,'ppt',0.101325,'MPa');   % The last argument is sat vapor pressure. It can be kept constant as suggested in SW_Diffusivity.m

D = cond/(avsigmat*cp); % Thermal diffusivity

BIN.Diff(i)=D;

visco = SW_Viscosity(avT,'C',avS,'ppt'); % dynamic viscosity mu

BIN.DynVisco(i)=visco;

kin_visco = visco/avsigmat; % kinematic viscosity nu

BIN.KinVisco(i)=kin_visco;

## STEP 12 – Vibration, inclination and acceleration flagging (resolve_turbulence)

### Vibration

% flagging for too much vibrations, flag if more than 5% of

% vibration per bin is >200 or if standard deviation is >100

% (i.e. 2 sigma C.L. = 95%)

Axi = Ax(jp);

Ayi = Ay(jp);

Axidev = nanstd(Axi);

Ayidev = nanstd(Ayi);

Aximean = abs(nanmean(Axi));

Ayimean = abs(nanmean(Ayi));

len_Axi = nansum(abs(Axi)>200); % Number of values with high Ax

len_Ayi = nansum(abs(Ayi)>200); % Number of values with high Ay

if len_Axi/length(jp) > 0.05

BIN.flag_vibration(i) = 1;

elseif (Axidev + Aximean) > 100

BIN.flag_vibration(i) = 1;

elseif len_Ayi/length(jp) > 0.05

BIN.flag_vibration(i) = 1;

elseif (Ayidev + Ayimean) > 100

BIN.flag_vibration(i) = 1;

else

BIN.flag_vibration(i) = 0;

end

### Inclination

% flagging for inclination, same as vibr for incl. > 5°

Incli = Incl_X(jps);

Inclidev = nanstd(Incli);

Inclimean = abs(nanmean(Incli));

len_Incli = sum(abs(Incli)>5);

if len_Incli/length(jps) > 0.05

BIN.flag_inclination(i) = 1;

elseif (Inclidev + Inclimean) > 2.5

BIN.flag_inclination(i) = 1;

else

BIN.flag_inclination(i) = 0;

end

### Acceleration

Only for Lake Garda so far:

%flagging for acceleration of instrument

%flag if acceleration is > 0.01 m/s/s (preliminary value, specific

%to lake Garda downward profiles, conservative estimate)

if strcmp(info.system,'Gar') & ismember(info.prof_dir,'down')

if abs(BIN.acc(i)) > 0.01

BIN.flag_acceleration(i) = 1;

else

BIN.flag_acceleration(i) = 0;

end

else

BIN.flag_acceleration(i) = NaN;

end

## STEP 13 – Shear spectral analysis (TKE_dis_spec)

Two theoretical spectra: Panchev & Kesich or Nasmyth (see Sanchez et al., 2011)

The arguments of the function TKE_dis_spec are listed in the table below.

Shear-based turbulence data processing might need to be modified according to the best practices from the ATOMIX project presented in Lueck et al. (2024).

| Argument name | Format | Units | Description | Value |
| --- | --- | --- | --- | --- |
| pres | N*1 | dbar | fast pressure vector within the bin | Pf(jp) in resolve_turbulence |
| x0 | N*2 | s-1 | despiked and filtered shear signals within the bin [S1,S2] | [sh1_hp(jp), sh2_hp(jp)] in resolve_turbulence |
| px0 | N*2 | counts | despiked vibration signals within the bin [Ax, Ay] | AAxy(jp,:) in resolve_turbulence |
| k1 | Float | cpm | minimum wavenumber for integration (small number) | param.info.minKS in load_parameters_lakename |
| k2 | Float | cpm | minimum wavenumber for integration (small number) | param.info.maxKS in load_parameters_lakename |
| fn | Float | Hz | maximum frequency for calculations | param.info.fAA in load_parameters_lakename |
| visco | Float | m2 s-1 | bin-averaged kinematic viscosity of water (ν) | kin_visco in resolve_turbulence |
| W | Float | dbar s-1 | bin-averaged profiling speed | WW in resolve_turbulence |
| sL | Integer | - | Length of FFT segments (number of samples) | Nfft in resolve_turbulence, computed from the number of FFT segments param.info.num_fft or equal to half the bin size by default |
| sOV | Integer | - | Length of the overlapping parts of the FFT segments (number of samples) | overlap in resolve_turbulence, equal to param.info.overlap or to 50% of the FFT segments by default |
| noise_corr | String | - | Type of denoising correction ('none' or 'Goodman') | param.info.noise_corr in load_parameters_lakename |
| sh_number | String | - | Shear probe for which eps_S is calculated ('sh1' or 'sh2') | 'sh1' or 'sh2' |
| plt | Boolean | - | Flag for making figure with profile and spectrum of the bin (False=no figure, True=plot spectum) | make_plot_spectra in resolve_turbulence (defined from bin indices with plots ind_plot_spectra) |
| presplt | N*1 | dbar | Pressure vector to plot on the profile figure | Pf(jp) in resolve_turbulence |
| Tplt | N*1 | °C | Temperature vector to plot on the profile figure | T1f(jp) in resolve_turbulence |

### Shear signal detrending

The shear signal x0=[S1,S2] and the vibration signal px0=[Ax,Ay] are both detrended with the Matlab built-in function detrend and only measurements with finite values for both S1 and S2 are kept:

%% Delete NaN and detrend

I = isfinite(x0); I=find(I(:,1)==1 & I(:,nc)==1); % generalized for the case with 2 sh

x0 = x0(I,:);

px0 = px0(I,:);

pres = pres(I);

x = detrend(x0,'linear');

px = detrend(px0,'linear');

I = isfinite(x); I=find(I(:,1)==1 & I(:,nc)==1);

x = x(I,:);

px = px(I,:);

pres = pres(I);

### Power spectral density computation

Set the parameters Fs (mean spatial sampling rate in pressure space, called rate in csd_odas), sL (length of the segments over which the Fast Fourier Transforms (FFT) are calculated, called nFFT in csd_odas) and sOV (overlapping length between FFT segments, called overlap in csd_odas) for the spectral computation:

%% Spatial resolution of the time series (cpm), FFT length and overlapping

Fs=length(pres)./ (max(pres)-min(pres)); % Mean sampling rate in pressure space [number of samples/dbar]

% FFT segment length (number of samples) if not specified (half the bin by

% default):

if sL == 0

sL = round(length(x(:,1))/2)-1;

end

% Number of samples shared between overlapping segments if not specified

% (50 % overlap by default)

if sOV == 0

sOV = round(sL/2);

end

Compute the autospectrum of the detrended shear signal (i.e., shear fluctuations) with the ODAS functions csd_odas (one shear vector) or csd_matrix_odas (shear matrix of two signals). These functions divide the shear signal into overlapping segments (Welch’s method), detrend each segment (with linear trend) , multiply the signal with a Hann window, compute the FFT for each segment (Matlab function fft) and average the obtained spectral densities between the segments. The returned cross-spectral density (PSD0) has units s-2/Hz.

%% Compute raw spectrum PSD0

if nc==1 % Only one shear sensor

% This function is provided by RSI in the ODAS libraries.

% Alternatively, pwelch can be used obtaining comparable results.

[PSD0,k] = csd_odas(x,x,sL,Fs,[],sOV,'linear');

elseif nc==2 % Two shear sensors

if strcmp(sh_number,'sh1')

column_index=1;

elseif strcmp(sh_number,'sh2')

column_index=2;

end

[PSD0,k,~,~] = csd_matrix_odas(x(:,column_index),x(:,column_index),sL,Fs,[],sOV,'linear');

end

### Denoised spectrum (Goodman)

Noise from high-frequency instrument vibrations is removed following the procedure from Goodman et al. (2006). The noise contribution to subtract to the measured shear spectrum is estimated from the transfer function H, defined as the ratio between the cross-spectrum of vibrations and shear to the auto-spectrum of vibrations H(k)=<XmeaskVk>/<Vk>2. This ratio comes from the linear contamination model xmeasz=xtruez+h(z)⋅v(z)  which is expressed as Xmeask=Xtruek+H(k)⋅V(k)  in wavenumber space. If two shear probes are used, the Goodman approach is applied by the ODAS function clean_shear_spec.

%% Compute denoised spectrum PSD

if strcmpi(noise_corr,'Goodman')

if nc==1

if sum(abs(px))>0

% Goodman denoising if only 1 sh is available.

% These functions are provided by RSI in the ODAS libraries.

% Alternatively, pwelch can be used obtaining comparable results.

% Auto-spectrum of vibration signal:

[PSDps, ~] = csd_odas(px,px,sL,Fs,[],sOV,'linear');

% Cross-spectrum of vibration and shear;

[CPS, ~, ~, ~] = csd_matrix_odas(x,px,sL,Fs,[],sOV,'linear');

%                 [PSDps, ~] = pwelch(px-nanmean(px),hann(sL),sOV,sL,Fs,'onesided');

%                 [CPS, ~] = cpsd(x-nanmean(x),px-nanmean(px),hann(sL),sOV,sL,Fs,'onesided');

H = CPS./PSDps; % Spectral density ratio

PSDcont = abs(H).^2.*PSDps;

PSD = PSD0 - PSDcont;

else

PSD = PSD0;

end

elseif nc==2

% Goodman denoising if 2 sh are available.

% This function is provided by RSI in the ODAS libraries.

[PSD, ~,~,~,~] = clean_shear_spec(px, x, sL, Fs);

PSD = permute(PSD, [3 1 2]);

PSD = squeeze(PSD(:, column_index, column_index));

end

elseif strcmpi(noise_corr,'none')

PSD = PSD0;

end

if nc==2

x=x(:,column_index);

end

### Spatial response correction

The loss of small-scale variance due to the spatial smoothing caused by the finite size of the probe is corrected by applying a transfer function defined in Macoun & Lueck (2003):

%% Apply spatial response correction

Hsr = 1./(1+(k./48).^2);

PSD = PSD./Hsr;

PSD0 = PSD0./Hsr;

### Iterative calculation of ε

### Lost variance correction

### Spectral fit to recalculate ε

### Median Absolute Deviation (MAD) calculation

%% Degrees of freedom and MAD threshold

NsegM = 2 * floor(length(x(:,1))/sL)-1;

dof = 1.9*NsegM; %according to ODAS 4.4

MADc = sqrt(2/dof)

%% Evaluation of MAD

MAD_S = mean( abs( (PSD(ik1:ik3)./NAS(ik1:ik3)) - mean(PSD(ik1:ik3)./NAS(ik1:ik3)) )  );

MADf = mean( abs( (PSD(ik1:ikn)./NAS(ik1:ikn)) - mean(PSD(ik1:ikn)./NAS(ik1:ikn)) )  );

fit_flag_S = 0;

if MAD_S<2*MADc

fit_flag_S = 1;

end

## STEP 14 – FP07 spectral analysis (gradT_dis_spec)

Two theoretical spectra: Batchelor or Kraichnan

### Temperature gradient detrending

%% Detrend

x = detrend(x0,'linear');

I=find(isfinite(x));

x=x(I);

pres=pres(I);

### Power spectral density computation

The power spectral density of the fluctuations (with respect to the bin average) of the detrended temperature gradient x=gradT [°C/m] is computed with csd_odas() using overlapping segments of length “sl”.

% This function is provided by RSI in the ODAS libraries.

% Alternatively, pwelch can be used obtaining comparable results.

[PSDT,fr] = csd_odas(x-nanmean(x),x-nanmean(x),sL,scalar_info.fs,[],sOV,'linear');

% [PSDT,fr] = pwelch(x-nanmean(x),hann(sL),sOV,sL,scalar_info.fs,'onesided');

Where PSDT [°C2.s.m-2] is the auto spectrum of x and fr  [s-1] are the corresponding frequencies ranging from 0 to the Nyquist frequency (half of the sampling frequency fs/2).

Segment size is not the same as Nfft (length of FFT)??

Increase in Nfft = less points in spectrum which is weird because nfft should be the number of freuncies in the spectrum! It seems that num_fft in the parameter is actually the number of segments and not the number of frequencies used for FFT.

- Higher num_fft = because segments used = lower resolution for spectral analysis because shorter segment but better accuracy because more spectra are averaged

The number of points in a segment is defined as Nfft (the actual number of segments is defined as (num_fft+1)/2) in resolve_turbulence and the overlap is by default half of Nfft:

### Conversion from time to space

The frequency spectrum is converting to the space domain (wavenumbers) using Taylor’s frozen hypothesis that states that temporal changes in temperature measured at one location are spatial changes in temperature that are advected by turbulent eddies at the “flow” velocity W (sinking speed of the profiler here). As a result, temporal frequencies f are directly related to wavenumbers k=f/W.

%% convert the frequency spectrum into the corresponding wavenumber spectrum (Taylor's frozen turbulence hypothesis)

PSDT=PSDT*W;

k = fr/W;

PSD=PSDT;

### Time response correction

Correct the spectrum based on the FP07 time response. Three methods can be used to determine the parameter τ that defines H from either single pole or double poles correction.

%% Time-response correction

if strcmp(time_corr,'KOC') % as in Kocsis et al. (1999)

tau = tau_0*W.^-0.5;

elseif strcmp(time_corr,'RSI') % as in Vachon and Lueck (1984)

F0 = 25*sqrt(W);

tau = (2*pi()*F0/sqrt(sqrt(2)-1))^(-1);

elseif strcmp(time_corr,'NAS') % as in Nash et al. (1999)

tau = tau_0*W.^-0.12;

elseif strcmp(time_corr,'SOM') % as in Sommer et al. (2013)

tau = 0.010;

else

error('Error: time_corr can be RSI, KOC or NAS')

end

%% Single or double correction

if strcmpi(npoles,'single')

H = 1./(1 + (2*pi()*tau*fr).^2);

H_lim=0.1;

elseif strcmpi(npoles,'double')

H = 1./(1 + (2*pi()*tau*fr).^2).^2;

H_lim=0.1;

end

PSD_raw=PSD;  % Raw spectrum

PSD = PSD./H; % Corrected spectrum

### Noise spectrum

The noise spectrum is computed with the ODAS function gradT_noise_odas() or with the function FP07noise().

%% Noise function (according to the function provided by RSI in the ODAS libraries)

noise_info=gradT_noise_odas;

noise_info.gamma_RSI = 1;

noise_info.E_n = 3e-9;

Sn = gradT_noise_odas(T_dT, T_string, W, fr, setupstr,noise_info); % Noise spectrum [(K/m)^2/Hz].

Sn = Sn*W;

Sn = Sn./H;

Sn(1) = 0;

% Alternatively, the noise can be computed as follows

% noisep = sensor dependent parameters. E.g. [-10.24,-0.89,fAA]

% where fAA = frequency of the anti-aliasing filter

% Sn = FP07noise(noisep,fr);

% Sn = Sn*W;

% Sn = Sn.*(2*pi()*k).^2;

% Sn = Sn./H;

% Sn(1) = 0;

### Integration range

The fitting is performed in the noise-free range of the spectrum.

Determines the initial range limits:

iknM0 = find(PSD<Snfact*Sn | H<H_lim | k==max(k),1,'first'); % Upper wavenumber limit of the fitting range

kS = ksfact*Pr^(-0.5)*kB_S; % Lower wavenumber limit according to Steinbuck et al. (2009)

k11 = max([kS,k1]);

ik11 = find(k>=k11,1,'first');

ikcor = ik11:iknM0;

Xiv = 6*D*sum( (PSD(ikcor(2:end))+PSD(ikcor(1:end-1))).*(k(ikcor(2:end))-k(ikcor(1:end-1))) )/2;

Xin = 6*D*sum( (Sn(ikcor(2:end))+Sn(ikcor(1:end-1))).*(k(ikcor(2:end))-k(ikcor(1:end-1))) )/2;

%% A signal-to-noise ratio larger than 1.3 was required in the integration range

if Xiv>1.3*Xin

cont = true;

Xiv = Xiv -Xin;

else

Xiv = 0;

Xi_T = 0;

end

### Calculate χST from shear

χST is computed by integrating the theoretical temperature spectrum (Kraichnan) using kB,S.

%% Calculates Xi using kB_S from shear probes and using the theoretical spectrum to correct for unresolved variance

% See eqs. (6) and (7) in the main text

if kB_S>0

BAT=Tspec(Tdis,Xiv,kB_S,k,D,q);

Xi_ref=6*D*sum( (BAT(ikcor(2:end))+BAT(ikcor(1:end-1))).*(k(ikcor(2:end))-k(ikcor(1:end-1))) )/2;

Xi_ST=Xiv.*Xiv./Xi_ref;  % See eq. (7) in the main text.

BAT=Tspec(Tdis,Xi_ST,kB_S,k,D,q);

%% Evaluation of MAD

MAD_ST = meanabsdev( PSD(ikcor), BAT(ikcor), Sn(ikcor) );

fit_flag_ST=0;

if MAD_ST<2*MADc

fit_flag_ST=1;

end

end

### Fitting (to get χT and εT)

Fitting is performed for wavenumbers ranging from max([kS, kuK]) to upper value defined from signal-to-noise ratio. The range limits are adapted during the fitting procedure, by doing three “searches”. The function cost_T_fit() is used for fitting the observed PSD to the theoretical spectrum. χT is computed in cost_T_fit() by integrating the measured spectrum with noise spectrum subtracted, similar to the computation of εS.

%% Fit the parameters in the noise free region

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

kF = k(iknM0);

ktest = linspace(max([5*k1,kF/5]),kF*5,40);

cost = nan(size(ktest));

Xif0 = nan(size(ktest));

dk = ktest(2)-ktest(1);

%% First search

for i = 1:length(ktest)

kS = k1;   % The Steinbuck et al. (2009) lower limit is not considered during the first round

kL = max([kS,k1]);

ikL = find(k>=kL,1,'first');

ikfit = ikL:iknM0;

[cost(i), Xif0(i),Steo] =  cost_T_fit(k, PSD, ktest(i), ikfit, Tdis, q, dof, Sn,D,Snfact);

end

LKHtest = - cost;

ML = max(LKHtest);

iML = find(LKHtest == ML);

kB00 = ktest(iML);

BATf0 = Tspec(Tdis,Xif0(iML),kB00,k,D,q);

kP=kB00/sqrt(6*q); % k at the peak

%% Look for the lower integration limit kL

if strcmp(int_range,'S')     % If only Steinbuck et al. (2009) condition

kLuk=k1; islope=2;

elseif strcmp(int_range,'L') % If only Steinbuck et al. (2009) + Luketina and Imberger (2001) conditions

% Find the intersection between observed and theoretical spectra

diffS = (PSD-BATf0)./abs(PSD-BATf0);

ikL = find(diffS<0,1,'first');

if ~isempty(ikL) && abs(PSD(ikL)-BATf0(ikL))>abs(PSD(ikL-1)-BATf0(ikL-1))

ikL=ikL-1;

end

% This was not mentioned in Luketina, it is a double check on

% the existence of an actual finestructure pattern.

% Calculate the slope of the first part of the spectrum using an increasing window

% Note: k(1) and PSD(1) = 0 --> the window starts from the 2nd point.

% The iteration starts from the 4th point to have at least 3 points to calculate the slope

ikP = find(k<=kP,1,'last');

slope=NaN(1,iknM0);

for i=4:iknM0

X=log10(k(2:i)); Y=log10(PSD(2:i));

X=[ones(length(X),1) (X)];

b=X\Y;     % \ computes the least square fit

slope(i)=b(2);

end

if ~isempty(ikL)

[vslope,islope]=nanmin(slope(1:min(ikP,ikL))); % find the minimum of the slope

if vslope>0 || nanmean(slope(1:islope))>0 % if the minimum slope is > 0, then

islope=2;

end

else

islope=2;

end

kLuk = min([k(islope),k(ikL),kP]);

end

%% Second search

cost = nan(size(ktest));

Xif0 = nan(size(ktest));

dk = ktest(2)-ktest(1);

for i = 1:length(ktest)

kS = ksfact*Pr^(-0.5)*kB00;    % Note: kS is based on the previous best

kL = max([kS,kLuk]);           % Note, if int_range=='L' here we consider the Luketina and Imberger (2001) condition

ikL = find(k>=kL,1,'first');

ikfit = ikL:iknM0;

[cost(i), Xif0(i)] =  cost_T_fit(k, PSD, ktest(i), ikfit, Tdis, q, dof, Sn,D,Snfact);

end

LKHtest = - cost;

ML = max(LKHtest);

iML = find(LKHtest == ML);

kB00 = ktest(iML);

BATf0 = Tspec(Tdis,Xif0(iML),kB00,k,D,q);

kP=kB00/sqrt(6*q); % k at the peak

%% Update the lower integration limit

if strcmp(int_range,'S')     % If only Steinbuck et al. (2009) condition

kLuk=k1;

elseif strcmp(int_range,'L') % If only Steinbuck et al. (2009) + Luketina and Imberger (2001) conditions

diffS = (PSD-BATf0)./abs(PSD-BATf0);

ikL = find(diffS<0,1,'first');

ikP = find(k<=kP,1,'last');

if ~isempty(ikL) && abs(PSD(ikL)-BATf0(ikL))>abs(PSD(ikL-1)-BATf0(ikL-1))

ikL=ikL-1;

end

if ~isempty(ikL)

[vslope,islope]=nanmin(slope(1:min(ikP,ikL))); % find the minimum of the slope

if vslope>0 || nanmean(slope(1:islope))>0 % if the minimum slope is > 0, then

islope=2;

end

else

islope=2;

end

kLuk = min([k(islope),k(ikL),kP]);

end

ikL = find(k>=kLuk,1,'first');

%% Third, refined search

MLp = -cost_T_fit(k, PSD, kB00+dk, ikfit, Tdis, q, dof, Sn,D,Snfact);

MLm = -cost_T_fit(k, PSD, kB00-dk, ikfit, Tdis, q, dof, Sn,D,Snfact);

deltak0=abs((2*dk)/sqrt(2*ML-MLm-MLp));

deltak = max([deltak0,dk]);

kmin2 = max([kB00-deltak,k(ikL+1)]);

kmax2 = kB00+deltak;

ktest = linspace(kmin2,kmax2,40);

clear cost Xif0

cost = nan(size(ktest));

Xif0 = nan(size(ktest));

for i = 1:length(ktest)

kS = ksfact*Pr^(-0.5)*kB00;    % Note: kS is based on the previous best

kL = max([kS,kLuk]);            % Note, if int_range=='L' here we consider the Luketina and Imberger (2001) condition

ikL = find(k>=kL,1,'first');

ikfit = ikL:iknM0;

[cost(i), Xif0(i)] =  cost_T_fit(k, PSD, ktest(i), ikfit, Tdis, q, dof, Sn,D,Snfact);

end

LKHtest = - cost;

MLKH = max(LKHtest);

iML = find(LKHtest == MLKH);

Xi_T = Xif0(iML);

kB = ktest(iML);

kP=kB/sqrt(6*q); % k peak

BATf = Tspec(Tdis,Xi_T,kB,k,D,q);

Finally, the dissipation rate is estimated from kB=12πεTνκT214 (Batchelor, 1959):

eps_T=visco*D^2*(2*pi()*kB)^4; % TKE dissipation rate

### Quality metrics

The quality metrics are computed and the data is flagged with fit_flag_T=1 if it is accepted.

- Mean Absolute Deviation (MAD):

Criterion to keep the data: MAD<2*MADc

- Likelihood Ratio (LR): Ratio between the likelihood from theoretical spectrum and likelihood power-law fitting: should be large. Criterion to keep the data: LR>2 (log10 space)

- Signal-to-Noise criterion?

- Confidence Interval?

%% Compute quality metrics

MAD_T = meanabsdev( PSD(ikfit), BATf(ikfit), Sn(ikfit) );

% Re-compute the integration range (for figure and fit_flag_T)

kS = ksfact*Pr^(-0.5)*kB;

kL = max([kS,kLuk]);

ikL = find(k>=kL,1,'first');

kL = k(ikL);

ikfit = ikL:iknM0;

% Calculates uncertainties in the fitting parameters

[MLp,Xip,~] = cost_T_fit(k, PSD, kB+deltak, ikfit, Tdis, q, dof, Sn,D,Snfact);

[MLm,Xim,~] = cost_T_fit(k, PSD, kB-deltak, ikfit, Tdis, q, dof, Sn,D,Snfact);

MLp = - MLp;

MLm = - MLm;

sKBT=abs((2*deltak)/sqrt(2*MLKH-MLm-MLp));

sXif = abs((Xip-Xim)/sqrt(2*MLKH-MLm-MLp));

% Fits to polynom (avoiding the noisy part)

ikfitA = ikfit(1):min([ikfit(end),find(BATf<Snfact*Sn,1,'first')]);

logK = log(k(ikfitA));

logS = log(PSD(ikfitA));

pp=polyfit(logK, logS,1);

Sm = exp(polyval(pp, log(k)));

LKHpol = -cost_MLE(PSD(ikfit), Sm(ikfit), dof, Sn(ikfit));

LR = MLKH - LKHpol;

LR = log10(exp(1))*LR;   % Likelihood ratio, see eq. (16)

kU=k(iknM0);

krange = log10(k(iknM0)) - log10(k(ikL));

fit_flag_T =0;

if LR>2 && MAD_T<MADc*2 ...

&& krange> 0.8 ...

&& kU>2*kP && kL<kP

fit_flag_T =1;

end

## STEP 15 – Spectral analysis flagging (resolve_turbulence)

Note that in resolve_turbulence.m and in the exported BINNED structure, flags are defined =1 in case of bad data (which is the opposite of gradT_dis_spec.m and TKE_dis_spec.m).

## STEP 16 – Vertical eddy diffusivity (resolve_turbulence)

The vertical eddy diffusivity Kz is estimated with two approaches:

- From ε, based on Osborn (1980): Kz=Γε/N2, where Γ≈0.2 is the mixing efficiency. The script provides estimate of Kz from shear (εS) and temperature (εT) measurements.

BIN.KOsborn_S1 = 0.2*BIN.eps_S1.*(BIN.N2).^-1;

BIN.KOsborn_T1=0.2*BIN.eps_T1.*(BIN.N2).^-1;

BIN.KOsborn_T2=0.2*BIN.eps_T2.*(BIN.N2).^-1;

BIN.KOsborn_S2 = 0.2*BIN.eps_S2.*(BIN.N2).^-1;

- From χ, based on Osborn & Cox (1972) : Kz=0.5 χ∂T∂z-2

BIN.KOsbornCox_T1 = 0.5*BIN.Xi_T1.*(BIN.avggradT1).^-2;

BIN.KOsbornCox_T2 = 0.5*BIN.Xi_T2.*(BIN.avggradT2).^-2;

BIN.KOsbornCox_ST1 = 0.5*BIN.Xi_ST1.*(BIN.avggradT1).^-2;

BIN.KOsbornCox_ST2 = 0.5*BIN.Xi_ST2.*(BIN.avggradT2).^-2;

Where the averaged temperature gradient is calculated in mean_grad() function:

BIN.avggradT(i) = mean_grad(Ps,sort_T_JAC,pres(i),max(Thorpe(i),info.dpD));   % SEB: evaluated on the resorted T profile, over the largest length scale between bin size and Thorpe length

## STEP 17 – Ozmidov length scale (resolve_turbulence)

The Ozmidov length scale LO=εN3 that represents the upper limit eddy size

in the inertial turbulent subrange is computed from εS:

BIN.LO_S1 = (BIN.eps_S1./BIN.N2.^(3/2)).^(0.5);

BIN.LO_S2 = (BIN.eps_S2./BIN.N2.^(3/2)).^(0.5);

# ADCP

# Wire-walker

# Moored thermistors

See Senafi et al. (2025)

# Eddy correlation

# Appendix A - Lists of variables for microstructure profilers

## Parameters

Parameters specified in the structure “param” in the function load_parameters_lakename are listed in the table below. Parameters without any default value need to be explicitly defined.

To continue:

| Parameter (alphabetical order) | Format | Unit | Description | Default value |
| --- | --- | --- | --- | --- |
| atm_press_method | String | - | Method used to correct the pressure data for atmospheric pressure. Either ‘offset’ (pressure offset), ‘min’ (minimum pressure in the file), 'cond' (using microconductivity spike, only for upward),'FP07' (using FP07 spike, only for upward) | ‘offset’ |
| cfgfile | String | - | Name of the configuration file (without .cfg extension) | ‘’ (empty) Need to be specified if modif_cfg=true in analyze_profiles |
| config | Structure | - | Probe configuration, with one field per microstructure channel (T1, T2, S1, S2, uC1, uC2). The value of each field is “true” if the probe is installed and “false” otherwise. | microCTD configuration: config.T1=true config.T2=true config.S1=true config.uC1=true |
| CTD_C | String | - | Name of the accurate conductivity sensor data exported by odas_p2mat (‘JAC_C’ for microCTD, ‘SBC’ for VMP) | microCTD: ‘JAC_C’ |
| CTD_Chl | String | - | Name of the fluorometer sensor data exported by odas_p2mat (‘Chlorophyll’ for microCTD, ‘FL’ for VMP) | microCTD: ‘Chlorophyll’ |
| CTD_T | String | - | Name of the accurate temperature sensor data exported by odas_p2mat (‘JAC_T’ for microCTD, ‘SBT’ for VMP) | microCTD: ‘JAC_T’ |
| CTD_Turb | String | - | Name of the turbidity sensor data exported by odas_p2mat (‘Turbidity’ for microCTD, ‘NTU’ for VMP) | microCTD: ‘Turbidity’ |
| filename_list | Cell array of strings | - | Names of data files (without .P extension) | - |
| folder | String | - | Path to the Level0 data folder | - |
| info | Structure | Different between parameters | Parameters for the data analysis, see more information in the table below. | See table below. |
| instrument | String | - | Name of the instrument (e.g., ‘microCTD’, ‘VMP’) | ‘microCTD’ |
| offset_P | Float | dbar | Pressure measured in the atmosphere, used to correct the pressure data if atm_press_method is set to ‘offset’ | 0 |
| S_sh1, S_sh2 | Float | V.m-2.s2 | Shear probe sensitivity provided on the calibration sheet | Not specified (only used if the configuration file is not specified) |
| SNname | String | - | Serial number | microCTD SN: ‘310’ |
| space_cfg | Boolean | - | Format of the configuration file: =true if there is a space between name of variables and values (microCTD), =false otherwise (VMP) | true |
| unit_Chl | String | - | Units of chlorophyll data | ‘ppb’ |
| unit_Turb | String | - | Units of turbidity data | ‘FTU’ |

Parameters specified in the structure “param.info” in the function load_parameters:

| Parameter (alphabetical order) | Format | Unit | Description | Default value |
| --- | --- | --- | --- | --- |
| despike_A | 1*3 array of double/floats | -, Hz, s | Triplet of parameters to remove spikes in accelerometer data (ODAS function “despike”). First value is the threshold for the signal ratio. Second value is the cutoff frequency of the low-pass smoothing filter. Third value is the duration of data to remove around a spike. | [8,0.5,0.04] |
| despike_sh | 1*3 array of double/floats | -, Hz, s | Triplet of parameters to remove spikes in shear data (ODAS function “despike”), similar to despike_A. | [8,0.5,0.04] |
| dp | Double/Float | m | Bin overlap | 0.5 |
| dpD | Double/Float | m | Bin size | 1 |
| fAA | Double/Float | Hz | Maximum frequency for calculations (also called fn), around 90% of the cut-off frequency of the anti-aliasing fi lter, which is instrument-specific (typically 98 Hz, called f_AA in ODAS), to avoid the slight attenuation near f_AA. fAA=0.9*f_AA=88.2 Hz | 90 |
| int_range | String (‘L’=Luketina and Imberger, 2001 or 'S'=Steinbuck, 2009) | - | Integration range | ‘L’ |
| k_HP_cut | Double/Float | cpm | Cutoff wavenumber based on low-frequency profiler motions | 0.85 (VMP) 0.5 (microCTD) |
| kmax_factor | Double/Float | - | Factor multiplying kn used to compute the maximum dissipation rate of TKE | 1/1.66 |
| ksfact | Double/Float | - | Coefficient for the upper bound of the inertial-convective subrange, as in Steinbuck et al. (2009) | 0.04 |
| maxKS | Double/Float | cpm | Maximum wavenumber for shear spectral integration | 14 |
| mindur_detect | Double/Float | s | Minimum duration of the profiles to detect | 30 |
| minKS | Double/Float | cpm | Minimum wavenumber for shear spectral integration (kmin) | 0.1 |
| minKT | Double/Float | cpm | Minimum wavenumber for temperature spectral integration (kmin) | 1 |
| minvel_detect | Double/Float | m/s | Minimum velocity of the profiles to detect | 0.1 |
| Nasmyth_spec | String (‘ODAS’ or ‘EPFL’) | - | Method for Nasmyth spectral fitting ? | ‘EPFL’ |
| noise_corr | String (none or ‘Goodman’) | - | Method for noise correction ? | ‘Goodman’ |
| noisep_T1, noisep_T2 |  |  |  | [-10.08+0.25,-0.97,param.info.fAA] |
| npoles | String (single' or 'double') | - | Type of FP07 time response correction | ‘single’ |
| num_fft | Integer | - | Number of FFT segments in each bin | 3 |
| overlap_pct | Double/Float | % | Percentage of overlap between FFT segments | 50 |
| peak_rem_T1, peak_rem_T2 |  |  |  | [0,0] |
| pmax | Double/Float | dbar | Maximum depth of the binned profiles | 30 |
| pmin | Double/Float | dbar | Minimum depth to start the profiles in the first profile detection algorithm | 0 |
| prof_dir | String (‘up’ or ‘down’) | - | Profiling direction | ‘down’ |
| q | Double/Float | - | Turbulent parameter (constant) in the theoretical temperature spectrum | 5.26 |
| Snfact | Double/Float | - | Maximum acceptable signal to noise ratio, as in Goto et al. (2016) | 1.55 |
| system | String | - | Name of the lake | ‘default’ |
| time_corr | String ('RSI', 'KOC'= Kocsis, 'NAS'=Nash et al., 1999 ) | - | FP07 time response correction method determining the parameter “tau” | ‘NAS’ |
| time_res | Double/Float | s | Nominal FP07 response time tau_0 (only used with ‘KOC’ and ‘NAS’ methods) | 0.0058 |
| Tspec | String (‘K’=Kraichnan or ‘B’=Batchelor) | - | Reference temperature spectrum to fit to the observations | ‘K’ |

## Level 1 data

The following variables are exported by odas_p2mat (see also Tables 2 & 3 in TN039; Ns is the length of vectors with slow frequency = 64 Hz, Nf is the length of vectors with fast frequency = 512 Hz):

| Variable name (alphabetical order) | Format | Unit | Description |
| --- | --- | --- | --- |
| Ax, Ay, Az | Nf*1 | counts | Piezo-accelerometer along the x, y and z-axes |
| C1_dC1, C2_dC2 | Nf*1 | counts | Raw microstructure conductivity with pre-emphasis* |
| C1_fast, C2_fast | Nf*1 | mS cm-1 | Microstructure conductivity time series at fast frequency |
| C1_slow, C2_slow | Ns*1 | mS cm-1 | Microstructure conductivity time series at slow frequency |
| cfgobj | Structure | - | Parsed configuration file |
| ch255 | Ns*1 | - | 7FF0 hexi-decimal, 32 752 Decimal |
| date | String | - | Date of the first measurement (format yyyy-mm-dd) |
| Day | Integer | days | Day number of the first measurement |
| filetime | Float | Days since 01.01.0000 | Time of the first measurement |
| FL or Chlorophyll | Ns*1 | ppb | Chlorophyll from fluorescence sensor |
| fs_fast | Float | Hz | Sampling frequency of fast channels, i.e. microstructure sensors (512 Hz) |
| fs_slow | Float | Hz | Sampling frequency of slow channels, i.e. CTD sensors (64 Hz) |
| fullPath | String | - | Path to the data file |
| Gnd | Ng*1 | - | Ground on first µASTP board? |
| Gnd_2 | Ns*1 | - | Ground on first µASTP board? |
| gradC1 gradC2 | Nf*1 | mS cm-1 dbar-1 | Vertical conductivity gradient |
| gradT1, gradT2 | Nf*1 | °C dbar-1 | Vertical temperature gradient |
| header | Numerical array | - | Header of every 1-second recor |
| header_version | Integer | - | Version of the header of every record in the .P file |
| Hour | Integer | h | Number of hours of the first measurement |
| Milli | Float | msec | Number of milliseconds of the first measurement |
| Minute | Integer | min | Number of minutes of the first measurement |
| Month | Integer | months | Month number of the first measurement |
| NTU or Turbidity | Ns*1 | FTU | Turbidity |
| odas_version | Float | - | Version of ODAS |
| P | Ns*1 | counts | Raw pressure without pre-emphasis |
| P_dP | Ns*1 | counts | Raw pressure with pre-emphasis |
| P_fast | Nf*1 | dbar | Pressure at fast frequency (extrapolated) |
| P_slow | Ns*1 | dbar | Pressure at slow frequency |
| SBC or JAC_C | Ns*1 | mS cm-1 | Conductivity from CTD sensor |
| SBT or JAC_T | Ns*1 | °C | Temperature from CTD sensor |
| Second | Integer | sec | Number of seconds of the first measurement |
| setupfilestr | String | - | Information from the setup file |
| sh1, sh2 | Nf*1 | s-1 | Shear time series |
| speed_fast | Nf*1 | dbar s-1 | Instrument speed at fast frequency (absolute value) |
| speed_slow | Ns*1 | dbar s-1 | Instrument speed at slow frequency (absolute value) |
| t_fast | Ns*1 numerical array | s | Time since the start of the measurements at fast frequency |
| t_fast_YD | Nf*1 | Days of the year | Time since the start of the measurements at fast frequency |
| t_slow | Nf*1 numerical array | s | Time since the start of the measurements at slow frequency |
| t_slow_YD | Ns*1 | Days of the year | Time since the start of the measurements at slow frequency |
| T1, T2 | Ns*1 | counts | Raw microstructure temperature time series without pre-emphasis |
| T1_dT1, T2_dT2 | Nf*1 | counts | Raw microstructure temperature time series with pre-emphasis |
| T1_fast, T2_fast | Nf*1 | °C | Microstructure temperature time series at fast frequency |
| T1_slow, T2_slow | Ns*1 | °C | Microstructure temperature time series at slow frequency |
| temperature_fast | Nf*1 | °C | Temperature used to calculate kinematic viscosity. Equals the data from the last FP07 sensor without calibration (e.g., T2 with two FP07) |
| time | String | - | Time of the first measurement (format HH:MM:SS) |
| vehicle_info | Structure | - | Vehicle-specific info (including profiling direction) returned by odas_p2mat . |
| W_fast | Nf*1 | dbar s-1 | Rate of change of pressure at fast frequency |
| W_slow | Ns*1 | dbar s-1 | Rate of change of pressure at slow frequency |
| Year | Integer | years | Year number of the first measurement |

*pre-emphasis: addition of the signal with its time derivative to boost high frequencies and increase the resolution/reduce the noise of short-term fluctuations (see TN02: Mudge & Lueck, 1993)

## Level 2 data

The variables included in the four cell arrays (BINNED, SLOW, FAST, DISS_QL) of the Level 2 .mat files are listed in the table below. In the variable names (first column), the expression sensor should be replaced with S1, S2, T1, and/or T2 as specified in the description (third column). The number of values N corresponds to the number of bins for BINNED and DISS_QL, and to f⋅Δt for SLOW and FAST, where f is the sampling frequency (64 Hz or 512 Hz) and Δt is the profile duration. Nsh, Ncond, Nscal and Nacc are the number of shear probes, microconductivity sensors, scalar sensors (FP07 + microconductivity) and piezo-accelerometers, respectively. Nfch and Nsch are the number of fast and slow channels, respectively.

| Variable name (alphabetical order) | Format | Unit | Description | Script where variable is computed | Storage group (B=BINNED, S=SLOW, F=FAST, D=DISS_QL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | B | S | F | D |
| A_x, A_y | N*1 numerical array | counts | High-frequency vibrations from the pair of piezo-accelerometers | odas_p2mat |  |  | ✓ |  |
| AA | 513* Nacc*Nacc*N numerical array | counts2.Hz-1 | Cross-spectrum of acceleration signals for each dissipation estimate. | quick_look with get_diss_odas |  |  |  | ✓ |
| AA_filt | N*1 numerical array | counts | High-frequency vibrations kept for noise filtering, either A_x or A_y with spikes removed using the despike_A parameter | resolve_turbulence |  |  | ✓ |  |
| acc | N*1 numerical array | dbar s-2 | Bin-averaged instrument acceleration | resolve_turbulence with mean_grad_time() | ✓ |  |  |  |
| AOA | Empty | ° | Maximum angle of attack of the velocity field, empty if 3D velocity info.UVW is not given as an argument of get_diss_odas | quick_look with get_diss_odas |  |  |  | ✓ |
| approx_bin_size | Float | m | Average bin size from the fixed bin duration and the mean profiler speed | analyze_profiles |  |  |  | ✓ |
| avggradsensor | N*1 numerical array | °C m-1 | Bin-averaged temperature gradient based on the data from sensors T1 and T2 (<∂T/∂z>) | resolve_turbulence with mean_grad() | ✓ |  |  |  |
| avggradT | N*1 numerical array | °C m-1 | Bin-averaged temperature gradient based on the data from the precise CT | resolve_turbulence with mean_grad() | ✓ |  |  |  |
| bin_overlap | Float | m | Bin overlap | parameter dp in load_parameters_lakename | ✓ |  |  |  |
| bin_size | Float | m | Bin size | parameter dpD in load_parameters_lakename | ✓ |  |  |  |
| chlorophyll | N*1 numerical array | ppb | Chlorophyll-a measured by the FT sensor | odas_p2mat + resolve_turbulence with pres_av() | ✓ | ✓ | ✓ |  |
| conductivity | N*1 numerical array | μS cm-1 | Water conductivity measured by the precise CT and matched to the response of the temperature sensor | compute_rho_salinity + resolve_turbulence with pres_av() | ✓ | ✓ |  |  |
| Data_fast | Nfch*N numerical array | Different for each raw | Bin-averaged data for each fast channel. | quick_look with get_diss_odas |  |  |  | ✓ |
| Data_slow | Nsch*N numerical array | Different for each raw | Same as Data_fast for the slow channels. | quick_look with get_diss_odas |  |  |  | ✓ |
| date | One cell with a string | - | Measurement date (format yyyy-mm-dd) | resolve_turbulence | ✓ | ✓ | ✓ |  |
| datenum | Float | Number of days since January 1, 0000 | Measurements starting time (UTC) | resolve_turbulence | ✓ | ✓ | ✓ |  |
| density | N*1 numerical array | kg m-3 | Water density computed from precise CT | compute_rho_salinity + resolve_turbulence with pres_av() | ✓ | ✓ |  |  |
| depth | N*1 numerical array | m | Water depth | correct_pressure + resolve_turbulence with pres_av() | ✓ | ✓ | ✓ |  |
| Diff | N*1 numerical array | m2 s-1 | Thermal diffusivity | resolve_turbulence | ✓ |  |  |  |
| direction | One cell with a string | - | Profiling direction: downward or upward | resolve_turbulence | ✓ | ✓ | ✓ |  |
| diss_length | Integer | - | The length of the bins (number of samples) used to estimate each dissipation rate. Different from the argument diss_length of quick_look which is the duration [s] of a bin! | quick_look with get_diss_odas |  |  |  | ✓ |
| dof_e | Nsh *N numerical array | - | Degrees of freedom in each dissipation estimate. It is the number of wavenumber points used to estimate epsilon, divided by the Nyquist bandwidth, times the diss_length (in samples). | quick_look with get_diss_odas |  |  |  | ✓ |
| dof_spec | Float | - | Degrees of freedom of the spectral estimates based on Nuttall (1971), i.e. dof_spec = 1.9*num_of_ffts. This is a constant value based on the diss_length and fft_length. | quick_look with get_diss_odas |  |  |  | ✓ |
| dtsec | N*1 numerical array | s | Time during profiling since the start of measurements | odas_p2mat (variable t_fast) |  | ✓ | ✓ |  |
| DynVisco | N*1 numerical array | kg m-1 s-1 | Dynamic viscosity | resolve_turbulence with SW_Viscosity() | ✓ |  |  |  |
| e | Nsh*N numerical array | m2 s-3 | TKE dissipation rate from each shear probe | quick_look with get_diss_odas |  |  |  | ✓ |
| eps_sensor | N*1 numerical array | m2 s-3 | TKE dissipation rate for sensors S1, S2, T1, and T2 | resolve_turbulence with TKE_dis_spec() (S1, S2) or directly from kB_sensor (T1, T2) | ✓ |  |  |  |
| epssensormax | N*1 numerical array | m2 s-3 | TKE dissipation rate for sensors T1, and T2 with kB replaced by kn⋅fkmax | resolve_turbulence | ✓ |  |  |  |
| F | 513*N numerical array | Hz | Frequencies of the cross-spectra for each dissipation estimates. | quick_look with get_diss_odas |  |  |  | ✓ |
| f_AA | Float | Hz | 90% of the anti-aliasing frequency. The anti-aliasing frequency is defined as f_AA=98 Hz (same name!) in quick_look | quick_look with get_diss_odas |  |  |  | ✓ |
| f_limit | Float | Hz | Maximum frequency for calculations, which replaces f_AA if f_AA>f_limit. The default value is infinity to use f_AA as the maximum frequency. | quick_look with get_diss_odas |  |  |  | ✓ |
| fast_list | 1*Nfch cell array of strings | - | Names of fast channels | quick_look |  |  |  | ✓ |
| fast_sensor | N*1 numerical array | s-1, °C | Shear and temperature time series for sensors S1 and S2, and T1 and T2, respectively |  |  |  | ✓ |  |
| fast_sensor_hp | N*1 numerical array | s-1 | Filtered shear time series for sensors S1 and S2, where low frequency noise above the cutoff wavenumber (k_HP_cut) has been removed | resolve_turbulence |  |  | ✓ |  |
| fft_length | Integer | - | The length of the fft-segments (number of samples) that were ensemble-averaged into a spectrum. | quick_look with get_diss_odas |  |  |  | ✓ |
| filename | One cell with a string | - | Name of the raw .p file containing the analyzed profile | analyze_profiles | ✓ | ✓ | ✓ |  |
| fit_order | Integer | - | Order of the polynomial fit to the shear spectra, in log-log space, that is used to estimate the spectral-minimum wavenumber. | quick_look with get_diss_odas |  |  |  | ✓ |
| flag_acceleration | N*1 numerical array | - | Acceptance based on instrument acceleration (acc): 0=accepted, 1=rejected (only for Lake Garda, NaN otherwise) | resolve_turbulence | ✓ |  |  |  |
| flag_inclination | N*1 numerical array | - | Acceptance based on inclination signal (Incl_X): 0=accepted, 1=rejected | resolve_turbulence | ✓ |  |  |  |
| flag_sensor | N*1 numerical array | - | Acceptance flag for ɛ and χ estimates for sensors S1, S2, T1, and T2 based on the quality metrics: 0=accepted, 1=rejected | resolve_turbulence with TKE_dis_spec() (S1, S2) and gradT_dis_spec() (T1, T2) | ✓ |  |  |  |
| flag_Ssensor | N*1 numerical array | - | Acceptance flag for χST estimates for sensors T1 and T2 based on the quality metrics: 0=accepted, 1=rejected | resolve_turbulence | ✓ |  |  |  |
| flag_vibration | N*1 numerical array | - | Acceptance based on vibration signal (Ax, Ay): 0=accepted, 1=rejected | resolve_turbulence | ✓ |  |  |  |
| FM | Nsh *N numerical array | - | A 'figure of merit' given by FM = mad*sqrt(dof_spec). This metric can be used for quality control of the dissipation estimates. | quick_look with get_diss_odas |  |  |  | ✓ |
| fraction_A | Nacc*1 numerical array | - | Fraction of data removed by the despike function on the accelerometer data. | quick_look |  |  |  | ✓ |
| fraction_C | Ncond*1 numerical array | - | Fraction of data removed by the despike function on the microconductivity data. | quick_look |  |  |  | ✓ |
| fraction_sh | Nsh*1 numerical array | - | Fraction of data removed by the despike function on the shear data. | quick_look |  |  |  | ✓ |
| fs_fast | Float | Hz | Fast sampling rate of the data (parameter) from odas_p2mat | quick_look with get_diss_odas |  |  |  | ✓ |
| fs_fast | Float | Hz | Slow sampling rate of the data (parameter) from odas_p2mat | quick_look with get_diss_odas |  |  |  | ✓ |
| grad_sensor | N*1 numerical array | °C m-1 | High frequency vertical temperature gradient for sensors T1 and T2 | odas_p2mat (variable gradsensor) |  |  | ✓ |  |
| grT12 | N*1 numerical array | °C m-1 | Bin-averaged temperature gradient, averaged between the sensors T1 and T2 | resolve_turbulence | ✓ |  |  |  |
| Incl_x, Incl_y | N*1 numerical array | ° | Instrument pitch (y) and roll (x) from inclinometer | odas_p2mat |  | ✓ |  |  |
| K | 513*N numerical array | cpm | Wavenumbers of the cross-spectra for each dissipation estimate | quick_look with get_diss_odas |  |  |  | ✓ |
| K_max | Nsh*N numerical array | cpm | Maximum wavenumber for each dissipation estimate | quick_look with get_diss_odas |  |  |  | ✓ |
| kB_sensor | N*1 numerical array | cpm | Batchelor wavenumber based on ɛ from sensors S1, S2, T1, and T2 | resolve_turbulence with gradT_dis_spec() (T1, T2) or directly from eps_sensor (S1, S2) | ✓ |  |  |  |
| KinVisco | N*1 numerical array | m2 s-1 | Kinematic viscosity | resolve_turbulence | ✓ |  |  |  |
| kL_sensor | N*1 numerical array | cpm | Lower integration wavenumber for sensors S1, S2, T1, and T2 | resolve_turbulence with TKE_dis_spec() (S1, S2) and gradT_dis_spec() (T1, T2) | ✓ |  |  |  |
| KOsborn_sensor | N*1 numerical array | m2 s-1 | Diapycnal diffusivity from ɛ for sensors S1, S2, T1, and T2, equation (21) in Piccolroaz et al. (2021) | resolve_turbulence | ✓ |  |  |  |
| KOsbornCox_sensor | N*1 numerical array | m2 s-1 | Diapycnal diffusivity from χ for sensors T1, and T2, equation (22) in Piccolroaz et al. (2021) | resolve_turbulence | ✓ |  |  |  |
| KOsbornCox_Ssensor | N*1 numerical array | m2 s-1 | Diapycnal diffusivity from χST for sensors T1, and T2, equation (22) in Piccolroaz et al. (2021) (using kB from shear probes) | resolve_turbulence | ✓ |  |  |  |
| kpeak_sensor (kP) | N*1 numerical array | cpm | Wavenumber corresponding to the peak of the fitted Kraichnan spectrum for sensors T1 and T2 | resolve_turbulence with gradT_dis_spec() | ✓ |  |  |  |
| krange_sensor | N*1 numerical array | cpm | Log10 of the wavenumber integration range for sensors T1, and T2 | resolve_turbulence with gradT_dis_spec() | ✓ |  |  |  |
| kU_sensor | N*1 numerical array | cpm | Upper integration wavenumber for sensors S1, S2, T1, and T2 | resolve_turbulence with TKE_dis_spec() (S1, S2) and gradT_dis_spec() (T1, T2) | ✓ |  |  |  |
| LO_sensor | N*1 numerical array | m | Ozmidov length scale based on ɛ for sensors S1, S2, equation (23) in Piccolroaz et al. (2021) | resolve_turbulence | ✓ |  |  |  |
| LR_sensor | N*1 numerical array | - | Likelihood ratio between theoretical and power-law MLE fitting for sensors T1 and T2, equation (16) in Piccolroaz et al. (2021) | resolve_turbulence with gradT_dis_spec() | ✓ |  |  |  |
| LT | N*1 numerical array | m | Thorpe scale computed from bin-averaged Thorpe displacements in slow density profiles, equation (19) in Piccolroaz et al. (2021) | resolve_turbulence with pres_av() | ✓ |  |  |  |
| LTavg | float | m | Profile-averaged Thorpe scale (including layers with LTc>0 only) | resolve_turbulence |  | ✓ |  |  |
| LTc | N*1 numerical array | m | Thorpe scale from slow density profile, averaged over layers with at least one overturn | resolve_turbulence |  | ✓ |  |  |
| LTusensor | N*1 numerical array | m | Thorpe scale computed from bin-averaged Thorpe displacements in fast temperature profiles (sensors T1 and T2), equation (19) of Piccolroaz et al. (2021) | resolve_turbulence with pres_av() | ✓ |  |  |  |
| mad | Nsh *N numerical array | - | Mean absolute deviation of the base-10 logarithm of the spectral values from the Nasmyth spectrum over the wavenumber range used to make each dissipation estimate. | quick_look with get_diss_odas |  |  |  | ✓ |
| MAD_sensor | N*1 numerical array | - | Mean Absolute Deviation between measured and reference spectra for sensors S1, S2, T1, and T2 | resolve_turbulence with TKE_dis_spec() (S1, S2) and gradT_dis_spec() (T1, T2) | ✓ |  |  |  |
| MAD_Ssensor | N*1 numerical array | - | Mean Absolute Deviation between measured and reference temperature gradient spectra for sensors T1 and T2, using kB from shear probes | resolve_turbulence with gradT_dis_spec() | ✓ |  |  |  |
| MADc | N*1 numerical array | - | Rejection threshold value of Mean Absolute Deviation, which depends on bin size only | resolve_turbulence with TKE_dis_spec() (S1, S2) and gradT_dis_spec() (T1, T2) (value is overwritten) | ✓ |  |  |  |
| maxgradT | N*1 numerical array | °C m-1 | Local maximum temperature smoothed gradient within a bin based on the data from the precise CT | resolve_turbulence | ✓ |  |  |  |
| method | Nsh *N numerical array | - | Method used to make each dissipation estimate. A value of 0 indicates the variance method and a value of 1 indicates the inertial subrange fitting method | quick_look with get_diss_odas |  |  |  | ✓ |
| N2 | N*1 numerical array | s-2 | Background Brunt-Väisälä frequency squared, equation (20) of Piccolroaz et al. (2021) | resolve_turbulence with mean_grad() | ✓ |  |  |  |
| Nasmyth_spec | 513* Nsh *N numerical array | s-2.cpm-1 | Nasmyth spectrum for each shear probe at each dissipation estimate | quick_look with get_diss_odas |  |  |  | ✓ |
| nu | N*1 numerical array | m2.s-1 | Kinematic viscosity at each dissipation estimate | quick_look with get_diss_odas |  |  |  | ✓ |
| overlap | Integer | - | The overlap of each dissipation estimate (number of samples). | quick_look with get_diss_odas |  |  |  | ✓ |
| P | N*1 numerical array | dbar | Bin-averaged pressure at each dissipation estimate, from variable P_fast in odas_p2mat | quick_look with get_diss_odas |  |  |  | ✓ |
| params | Structure | Different for each parameter | Parameters used and returned by odas_p2mat. Parameters used by odas_p2mat. See ODAS Manual 4.4, p.117-120 for more details. | quick_look with odas_p2mat() |  |  |  | ✓ |
| pass_count_A | Nacc*1 numerical array | - | Number of passes of the despike function used to remove the accelerometer spikes. | quick_look |  |  |  | ✓ |
| pass_count_C | Ncond*1 numerical array | - | Number of passes of the despike function used to remove the microconductivity spikes. | quick_look |  |  |  | ✓ |
| pass_count_sh | Nsh*1 numerical array | - | Number of passes of the despike function used to remove the shear spikes. | quick_look |  |  |  | ✓ |
| piezo | Integer | - | =1 if piezo accelerometer data was detected (e.g., microCTD), =0 otherwise (e.g., VMP). | quick_look |  |  |  | ✓ |
| pressure | N*1 numerical array | dbar | Water pressure | odas_p2mat (variable P_slow for S and P_fast for F), resolve_turbulence for B | ✓ | ✓ | ✓ |  |
| profID | Integer | - | Profile ID | analyze_profiles (variable counter) | ✓ | ✓ | ✓ |  |
| profiles_in_this_file | Integer | - | Number of profiles detected in the file. | quick_look |  |  |  | ✓ |
| ql_info | Structure | Different for each parameter | Parameters used to process the data by quick_look. See ODAS Manual 4.4, p.137-141 for more details. | quick_look |  |  |  | ✓ |
| ql_info_in | Structure | Different for each parameter | Input parameters of quick_look. See ODAS Manual 4.4, p.137-141 for more details. | analyze_profiles |  |  |  | ✓ |
| raw_pressure | N*1 numerical array | dbar | Raw pressure time series before any correction for atmospheric pressure | analyze_profiles (variables data_prof.P_fast_raw and data_prof.P_slow_raw) |  | ✓ | ✓ |  |
| salinity | N*1 numerical array | mg l-1 | Water salinity, equation (18) of Piccolroaz et al. (2021) | compute_rho_salinity + resolve_turbulence for conversion | ✓ | ✓ |  |  |
| scalar_info | Structure | Different for each parameter | Processing parameters to compute scalar spectra with get_scalar_spectra_odas. See ODAS Manual 4.4, p.94-95 for more details. | quick_look |  |  |  | ✓ |
| scalar_spectra | Structure | Different for each parameter | Results of the scalar spectra calculation. See ODAS Manual 4.4, p. 96 for more details. | get_scalar_spectra_odas |  |  |  | ✓ |
| scalar_vector_list | 1*Ntemp cell array of strings | - | Names of the scalar vectors. | quick_look |  |  |  | ✓ |
| sh | 513* Nsh*Nsh*N numerical array | s-2.cpm-1 | Same as sh_clean but without cleaning by the Goodman coherent noise removal algorithm. | quick_look with get_diss_odas |  |  |  | ✓ |
| sh_clean | 513*Nsh*Nsh*N numerical array | s-2.cpm-1 | Wavenumber cross-spectrum for each shear probe signal at each dissipation estimate | quick_look with get_diss_odas |  |  |  | ✓ |
| sigmatsensor | N*1 numerical array | kg m-3 | Water density computed from sensors T1 and T2 and from extrapolated conductivity to fast frequency | compute_rho_salinity (variables data_prof.rhoT1S and data_prof.rhoT2S) |  |  | ✓ |  |
| slow_list | 1*Nsch cell array of strings | - | Names of slow channels | quick_look |  |  |  | ✓ |
| SPECTRUMsensor | 1*N structure | - | Spectrum parameters for each bin, for sensors T1 and T2 | resolve_turbulence with gradT_dis_spec() | ✓ |  |  |  |
| speed | N*1 numerical array | dbar s-1 | Bin-averaged absolute profiler speed, from variable speed_fast in odas_p2mat | quick_look with get_diss_odas() |  |  |  | ✓ |
| speed | N*1 numerical array | dbar s-1 | Bin-averaged vertical profiling speed | odas_p2mat (variable speed_slow) | ✓ |  |  |  |
| speed_source | String | - | Source of the speed of profiling. | quick_look |  |  |  | ✓ |
| spikes_A | 1*Nacc cell array | - | Indices to the spikes located in the signals from the accelerometers. | quick_look |  |  |  | ✓ |
| spikes_C | 1*Ncond cell array | - | Indices to the spikes located in the signals from the microconductivity probes. | quick_look |  |  |  | ✓ |
| spikes_sh | 1*Nsh cell array | - | Indices to the spikes located in the signals from the shear probes. | quick_look |  |  |  | ✓ |
| T | N*1 numerical array | °C | Bin-averaged temperature at each dissipation estimate, from variable temperature_fast in odas_p2mat | quick_look with get_diss_odas |  |  |  | ✓ |
| t | N*1 numerical array | s | Bin-averaged time at each dissipation estimate, from variable t_fast in odas_p2mat | quick_look with get_diss_odas |  |  |  | ✓ |
| temperature | N*1 numerical array | °C | Water temperature measured by the precise CT | odas_p2mat + resolve_turbulence with pres_av() | ✓ | ✓ |  |  |
| ThermCond | N*1 numerical array | W/m K | Bin-averaged thermal conductivity | resolve_turbulence with SW_Conductivity() | ✓ |  |  |  |
| time | One cell with a string | - | Measurement starting time (UTC, format HH:MM:SS) | resolve_turbulence | ✓ | ✓ | ✓ |  |
| turbidity | N*1 numerical array | FTU | Turbidity measured by the FT sensor | odas_p2mat + resolve_turbulence with pres_av() | ✓ | ✓ | ✓ |  |
| UA | 513* Nsh*Nacc*N numerical array | s-1.counts.Hz-1 | Cross-spectrum of shear probe and acceleration signals for each dissipation estimate. | quick_look with get_diss_odas |  |  |  | ✓ |
| upward | Integer | - | =1 if upward direction, =0 if downward direction | resolve_turbulence | ✓ | ✓ | ✓ |  |
| vehicle_info | Structure | Different for each parameter | Vehicle-specific info (including profiling direction, angle of attack, etc.) returned by odas_p2mat. | quick_look with odas_p2mat() |  |  |  | ✓ |
| velocity | N*1 numerical array | dbar s-1 | Vertical profiling speed | odas_p2mat (variable W_slow for S, variable W_fast for F) |  | ✓ | ✓ |  |
| WW | N*1 numerical array | dbar s-1 | Bin-averaged profiling speed (not smoothed) | resolve_turbulence | ✓ |  |  |  |
| Xi_sensor | N*1 numerical array | °C2 s-1 | Temperature variance dissipation rate for sensors T1 and T2, computed using MLE spectral fitting | resolve_turbulence with gradT_dis_spec() | ✓ |  |  |  |
| Xi_Ssensor | N*1 numerical array | °C2 s-1 | Temperature variance dissipation rate for sensors T1 and T2, computed using kB from shear probes | resolve_turbulence with gradT_dis_spec() | ✓ |  |  |  |
| Xiv1, Xiv2 | N*1 numerical array | °C2 s-1 | Temperature variance dissipation rate for sensors T1 (Xiv1) and T2 (Xiv2), computed using spectral integration in the well resolved part | resolve_turbulence with gradT_dis_spec() | ✓ |  |  |  |

+ variables sXIf1, sXif2, xKB1, sKB2, LKH1 removed because initialized but not computed in the scripts